AG-3

Cyber Resilience for an Agribusiness Back Office

AG3 Detailed Network Design

COIT20265 Networks and Information Security Project

Group PG5

CQUniversity Australia

Table of Contents

1 Purpose and scope	4

1.1	Implementation	4

1.2	Meaning of implementation status	4

2 Prototype environment	4

2.1 Systems and address plan	4

2.2 Software and local capacity	5

2.3 Prototype network layout	5

2.4 Conceptual information mapping and implemented data flow	6

2.5 Prototype architecture summary	8

2.6 Prototype implementation requirements	8

3 Implementation plan	9

3.1 Coordination and change control	9

3.2 Build decisions	10

3.2.1 Implementation challenges encountered	10

4 Preparation guide	11

4.1 Record the working baseline	11

4.2 Guide baseline and substitution	12

5 Network implementation guide	12

5.1 Configure pfSense	12

5.2 Required application paths	13

5.3 Configure host VLAN interfaces	13

6 APP01 implementation guide	14

6.1 Install the application stack	14

6.2 Configure access and application settings	15

7 MON01 implementation guide	16

7.1 Install the Wazuh central components	16

7.2 Enrol APP01	17

7.3 Configure monitoring inputs	17

8 BKP01 implementation guide	18

8.1 Configure MinIO storage	18

8.2 Verify effective permissions	19

8.3 Connect Restic on APP01	19

8.4 Capture a consistent Nextcloud backup	20

8.5 Automate the backup and retention	20

9 Implementation completion checks	21

10 Controlled restore implementation procedure	22

11 Troubleshooting guide	23

12 Handover and production limitations	24

12.1 Handover requirements	24

12.2 Production limitations and improvements	25

References	26

How this document structured

The scope and the environment and implementation sequence are addressed in Sections 1–3. The setup and completion of implementation is detailed in sections 4-9. The technical controlled-restore procedure is described in Section 10 and troubleshooting and handover are described in Sections 11 and 12. On completion of each stage the operator checks it off and proceeds to the next stage.

AG3 Firewall, Segmentation and Access Control Design combines the firewall design, segmentation and access-control design and the firewall-specific acceptance evidence. The consolidated prototype testing and validation evidence is contained in AG3 Prototype Testing and Validation Report. Incident roles, containment decisions and service-release approval are contained in the incident and recovery plan. Only checks required for each stage of implementation are included in this guide.

# 1 Purpose and scope

The AG3 prototype is the primary cyber resilience process: Users store their synthetic business files on APP01, Wazuh on MON01 receives a subset of security events, and Restic on APP01 encrypts backup files and pushes them to MinIO on BKP01. pfSense regulates traffic between the logical zones. The implementation plan and guide aids the team member building, rebuilding or assuming this lab. Specifies the sequence of work, the values to be used for the configuration, and the results to be returned for each step. A working file portal and monitoring connection/recovery copy is necessary prior to the final demonstration.

  - Implementation boundary

The lab is composed of one firewall, the Office and Management clients, and three service hosts. Student hardware is used for existing student virtual machines on Ubuntu. Only synthetic financial records, supplier information, staff examples and operational files are stored in Nextcloud. Production availability and switch-port isolation is not provided by the prototype.

  - Meaning of implementation status

Designed means that the set-up or process is described. Implemented - it is configured. Validated: An executed check has supporting evidence. Pending – work and/or evidence is not complete. These terms are for the final configuration, as a change later in the project could affect an earlier test.

Designed means that the set-up or process is described. Implemented - it is configured. Validated: An executed check has supporting evidence. Pending – work and/or evidence is not complete. These terms are for the final configuration, as a change later in the project could affect an earlier test. The earlier implementation record reports application, monitoring and backup work. The executed firewall matrix FT-01 - FT-15 and the firewall specific evidence is provided in AG3 Firewall, Segmentation and Access Control Design. AG3 Prototype Testing and Validation Report is the one-stop repository for all tests, application validation, monitoring verification, backup and recovery results for prototype testing. Service claims in this guide only reflect the actual configuration and are not considered formal validation without the actual evidence in these artefacts

# 2 Prototype environment

## 2.1 Systems and address plan

| System | Purpose | Address and gateway |
| --- | --- | --- |
| pfSense | VLAN gateways and stateful firewall | Each VLAN gateway is 10.20.VLAN.1/24 |
| Office client | Nextcloud user access in VLAN 10 | 10.20.10.10/24; gateway 10.20.10.1 |
| APP01 | Nextcloud, Wazuh agent and Restic in VLAN 20 | 10.20.20.10/24; gateway 10.20.20.1 |
| MON01 | Wazuh manager, indexer and dashboard in VLAN 30 | 10.20.30.10/24; gateway 10.20.30.1 |
| BKP01 | MinIO object storage in VLAN 40 | 10.20.40.10/24; gateway 10.20.40.1 |
| ADMIN-PC | Trusted administration in VLAN 99 | 10.20.99.10/24; gateway 10.20.99.1 |
| Recovery LAN | Temporary local firewall administration | pfSense 192.168.50.1/24; client uses an unused address |

The last firewall configuration has WAN as ue0 and DHCP, recovery LAN as re0, and VLAN children as re0.10, re0.20, re0.30, re0.40 and re0.99. These names are unique to this firewall. A replacement device can have a different set of interface names and Akib records those prior to assigning addresses

## 2.2 Software and local capacity

pfSense offers routing and firewall rules. The file portal and role permissions are provided by Nextcloud. Wazuh provides monitoring. Restic takes snapshots and stores repository objects in MinIO, which are then encrypted. APP01 was created as an Ubuntu Server 24.04.4 ARM64 virtual machine running on VMware Fusion Pro 26.5.2 on an Apple M1 Macbook, 2 vCPU, 4GB RAM and a 40GB virtual disk. Other service hosts keep the virtualisation platform registered by its owners.

Rebuild is based upon the approved releases recorded from the working machines. Prior to a clean install, the exact releases and VM allocations are recorded in the build record. The operator also inspects the disk space available to the working files, repository and monitoring indices and an alternate restore location.

The vendor quickstart recommends 4 vCPU, 8 GiB RAM and 50 GB storage for a new small Wazuh all-in-one host with 1-25 agents. This is a planning reference and is not a measured specification of the existing MON01. APP01 and BKP01 allocations are based on the synthetic data set and the measured free space (Wazuh n.d.a).

## 2.3 Prototype network layout

Figure 1 AG3 prototype network and logical zones

The provided figure is preserved to indicate the five logical zones and physical connection. TP-Link LS108G is an unmanaged switch. It does not control what port is associated with a VLAN and an endpoint that can modify its tag can access another logical zone. In the lab, it thus shows a routed firewall behaviour in a controlled environment.

Temporary recovery LAN is used for local administration and recovery of access to the firewall. It should not offer an alternate path to the VLAN firewall for a service host. Both the WAN and LAN side of the firewall must also be different and non-overlapping subnets.

This is a general figure of Wazuh monitoring intentions. Limitations: the actual coverage may be only the configured sources and demonstrated events. It is not a proof of collection of all activities, all traffic behaviours or every backup and restore operation.

Required network condition

Any prototype zone traffic is routed to the other prototype zone via a separate VLAN gateway per service host. The operator checks the route and sees the respective traffic at pfSense. If a successful connection is made via a host-only or WiFi or secondary NAT path, then the intended VLAN rule is not validated. Conceptual information mapping and implemented data flow.

## 2.4 Conceptual information mapping and implemented data flow

Figure 2 Supplied conceptual information mapping for Plains Pastoral Co.

Figure 2 was created at the Design phase to illustrate the intended relationships between the Office, application, monitoring and backup functions. It is not kept as a true “as-built” record of the final prototype, but rather as a conceptual information map. Some paths were further evolved during the implementation as the team overcame connectivity and monitoring challenges.

In the prototype that was implemented, APP01 is communicating with MON01 in VLAN 30 via the authorized Wazuh communication path. APP01 also sets up the backup connection to BKP01, using Restic and MinIO. Backup success/failure is checked from APP01 and not by direct BKP01 monitoring. Instead, direct BKP01 monitoring was included in the previous concept but was not included in the monitoring approach demonstrated.

Also included is a limited backup-account concept and central pfSense logs to Wazuh. In the implementation phase, the backup account was a restricted Restic service account that is only allowed to perform certain repository operations required for a normal backup and restore. The logging of central pfSense was part of the monitoring concept, but was not successfully shown in the final prototype. The actual behaviour is determined by the AG3 Firewall Segmentation and Access Control Design, AG3 Security Monitoring and Wazuh Implementation and AG3 Prototype Testing and Validation Report.

## 2.5 Prototype architecture summary

The prototype is composed of one central pfSense firewall, and five logical security zones. The following table outlines the elements that were put in place prior to the start of the guide. It should be read in conjunction with Figure 1 and conceptual information mapping in Figure 2. Where the conceptual view has been developed during the project, the written implementation sections describe the final as-built behaviour.

| Component | Prototype function | Zone / address | Primary service |
| --- | --- | --- | --- |
| pfSense | Routing, VLAN gateways and inter-zone firewall | VLANs 10, 20, 30, 40 and 99 | Stateful firewall and NAT |
| Office client | Normal user access to company file service | VLAN 10 / 10.20.10.10 | HTTPS client |
| APP01 | Business file service, Wazuh agent and backup source | VLAN 20 / 10.20.20.10 | Nextcloud, Restic |
| MON01 | Central security monitoring | VLAN 30 / 10.20.30.10 | Wazuh manager, indexer and dashboard |
| BKP01 | Separated backup repository | VLAN 40 / 10.20.40.10 | MinIO |
| ADMIN-PC | Trusted administration source | VLAN 99 / 10.20.99.10 | Approved management access |

The prototype architecture is purposefully smaller than the target production architecture. The unmanaged switch, single firewall and single service instances are laboratory constraints, and the security functions and allowed communication paths are based upon the chosen AG3 design.

## 2.6 Prototype implementation requirements

The implementation procedure is controlled by the following requirements. The identifiers are used throughout the guide to indicate the reason for a configuration step. There is no Formal PASS or FAIL results assigned here; executed validation should be provided in AG3 Prototype Testing and Validation Report, unless validation evidence is specific to a particular firewall and is stored in AG3 Firewall Segmentation and Access Control Design.

| ID | Implementation requirement |
| --- | --- |
| IR-01 | Office users must reach APP01 using HTTPS through the intended pfSense path. |
| IR-02 | Normal Office access must not provide direct access to BKP01 or privileged management services. |
| IR-03 | APP01 must be able to enrol with MON01 and send the monitoring events selected for the prototype. |
| IR-04 | APP01 must send the required Nextcloud backup set to BKP01 using the approved Restic and MinIO path. |
| IR-05 | The backup set must include the database, application configuration and required data captured from one controlled consistency point. |
| IR-06 | A selected backup must be restorable to a separate staging or recovery location without overwriting the only live copy. |
| IR-07 | Privileged administration must use approved management or retained local recovery access rather than normal Office access. |
| IR-08 | Live passwords, repository secrets, TOTP recovery codes and service keys must not be stored in the shared report or GitHub repository. |
| IR-09 | Prototype limitations caused by the unmanaged switch, self-signed certificates or unverified transport must be stated rather than treated as production controls. |

# 3 Implementation plan

Each system is built individual and then the necessary paths are connected. Faults are resolved locally with this order: application fault, monitoring and backup. Stages are achieved in relation to the dependency and acceptance condition below, dates are noted of when the work is done. The coordination and change control process.

| Stage and owner | Depends on | Completion condition |
| --- | --- | --- |
| 1 Preparation All owners | Approved scope and hardware | Build versions, paths, credentials, addressing and rollback access recorded |
| 2 Network Akib | Preparation | Host gateways and required routed paths work; unrelated paths remain blocked |
| 3 APP01 Shourab | Network | HTTPS file access, role permissions and administrator TOTP checked |
| 4 MON01 Tanvi with Shourab | APP01 and network | APP01 enrolled and fresh test events received with correct identity |
| 5 BKP01 Faysal | Network | MinIO service, bucket and effective service-account permissions checked |
| 6 Backup integrations Faysal with Shourab | APP01 and BKP01 | Consistent snapshot, job record and controlled restore completed |
| 7 Integrated handover All owners | All earlier stages | Evidence, configuration exports and remaining limitations reconciled |

## 3.1 Coordination and change control

Akib is in charge of firewall and network alterations. All the APP01 and application paths are controlled by Shourab. Tanvi manages the monitoring inputs and evidence of the agents. Backing up target, credentials and repository maintenance is done by Faysal. Prior to any change is to be made, it is agreed with the owner of another system that will be impacted by the change.

Change record includes reason, affected host, previous setting, new setting, owner information and rollback method. The owner saves the corresponding setup prior to its editing and repeats the corresponding setup check after editing. The team keeps track of the work done and any outstanding work in its project board.

Recorded change example

The APP01 path-MTU problem is also kept as an example to the change process. It indicates that the team modified a single parameter that was observed and also re-applied the completion check after the parameter was modified instead of implementing a blanket network change.

| Item | Recorded value |
| --- | --- |
| Affected host | APP01 integration interface enp26s0 |
| Observed problem | TCP 443 connected from the Office client, but the Nextcloud HTTPS request stalled. |
| Previous setting | MTU 1500 |
| Changed setting | MTU 1400 |
| Reason | To correct the demonstrated transfer failure on the integrated lab path. |
| Owner / coordination | Shourab with Akib for the APP01 and network path. |
| Rollback | Return enp26s0 to MTU 1500 if the change does not correct the fault. |
| Completion check | Office-to-APP01 browser request through pfSense was repeated after the change; formal evidence is retained in AG3 Prototype Testing and Validation Report. |

## 3.2 Build decisions

It is a push model, similar to the current integration model where Restic is served on APP01 and MinIO on BKP01. The design separates the backup storage from the usual Office storage but APP01 still stores a repository password and storage credential. There are two improvements that are required: segregation and trusted recovery administration for the destructive maintenance.

### 3.2.1 Implementation challenges encountered

Initial Wazuh simulation- Tanvi initially tried out Wazuh using Kali Linux virtual machines on Oracle VirtualBox. Managing multiple virtual adapters and identifying the correct interface proved tricky in the early simulation, but failed-login detection, file integrity monitoring and vulnerability detection/CIS assessment did work.

VLAN 30 connectivity- Issues of primary concern were the connectivity of MON01 on VLAN 30. VLAN ID 0 enabled general connectivity, however VLAN 30 gateway was not available, and VLAN ID 30 was able to reach 10.20.30.1, but had an impact to Internet and cross network access on the Windows host. The trouble was found to be due to the combination of host VLAN tagging, VirtualBox bridging and the available switch configuration.

Migration to macOS- MON01 was relocated to a Mac as an Ubuntu virtual machine, due to the lack of reliable connectivity on the Windows host. Wazuh was reinstalled and the network configuration was repeated, proceeding with integration.

Cross-device agent deployment- APP01 was on one team member's Mac and MON01 on another, thus Wazuh agent enrolment was reliant upon pfSense routing, firewall paths (1514, 1515) and addressing. Previously, enrollment problems were corrected by reviewing the route and service ports prior to redeployment of the APP01 agent.

BKP01 monitoring change- During implementation, direct monitoring of BKP01 to Wazuh was decreased due to the need for adding routing and configuration complexity to it. This demonstrated monitoring path, in contrast, tracks backup success and failure records created on APP01. This provides less direct visibility into BKP01, which is kept as a prototype limitation.

Project-specific detections- Not all the AG3 events needed for the demo were represented on default Wazuh behaviour. The team thus set up some monitored directories and configured failed logins, backup results and ransomware-type file activity handling events, and reserved the final detection claims to strictly the events that were actually shown.

# 4 Preparation guide

## 4.1 Record the working baseline

The working baseline information includes system identity, addressing, routes, time synchronising and descriptions of installed software versions per host and of free disk space.

Identify and label the WAN and LAN ports of the firewall. Before changing interfaces, make sure there is local console access and export the pfSense configuration.

For each VM: be sure to write down its host machine, OS type, CPU architecture, memory, disk allocation, virtual adapter mode, physical adapter and VLAN tagging location. Check if the selected software is available for that operating system and architecture.

Record the following information of the installed software: Version, Application directory, Data directory, Database name, MinIO Bucket, Backup script, Schedule, Restore location. Keep the same set of installation files or approved download locations and their integrity information.

Use separate identities for administrators and services. Keep all passwords, TOTP recovery codes and repository password in secure places. Live secrets are not in shared reports or GitHub.

Check for DNS resolution, time synchronisation, free disk space. Avoid installing and upgrading software during outside of the planned change window. Keep a clean VM checkpoint to revert to where appropriate and keep a separate data back up.

Version of Nextcloud and data directory is checked on APP01, and Wazuh package versions are recorded on MON01. Service names and their installation paths are based on that of actual deployment. 4.2 Guide baseline and substitution

## 4.2 Guide baseline and substitution

The examples in the application section are based on manual installation of Ubuntu with Apache, MariaDB and /var/www/nextcloud. For example, the directory for the data is /srv/nextcloud-data and the database is nextcloud. The supported paths and service locations of a Snap, container or different web-stack installation are used. These differences are resolved by the operator before embarking on following the guide.

Persistent network changes are done via the local console or by using a tested recovery connection. It deploys netplan try before attempting to apply a network change on the Linux OS, which means that an unconfirmed remote change can be rolled back (Canonical n.d.).

# 5 Network implementation guide

## 5.1 Configure pfSense

Setup and/or restore the approved pfSense build. Assign WAN to the upstream router, and LAN to the prototype switch. Verify that their subnets aren't overlapping. Keep temporary local administration path.

Open Interfaces > Assignments > VLANs. Add VLAN IDs 10, 20, 30, 40 and 99 on the LAN parent interface. Create each VLAN as an interface, enable it and assign a static .1 address for each in a /24.

On internal VLAN interfaces, leave Block private networks disabled. Check the WAN setting with the upstream network; a WAN on a private home-router should be set-up specifically, and not as a blanket rule for all interfaces.

Create aliases for APP01, MON01, BKP01, ADMIN-PC and the internal networks. Only add the necessary service rules to the interface from which the connection is coming from. Discuss floating & group rules about exception.

Configure the permitted DNS and NTP access. Confirm outbound NAT – This will validate the VLANs that need access to be installed or updated. Eliminate temporary broad allow rules after needed services are provided.

Test trustworthy management access prior to limiting the temporary recovery LAN. Keep the last export for the configuration and also keep console access in case the rule for management fails.

The rules on an interface govern traffic flowing into that interface. It is not necessary to have a second broad reverse rule, since a matching allow rule is created that allows replies. This is crucial for Restic restores as APP01 still triggers the API connection (Netgate, n.d.a, n.d.b).

## 5.2 Required application paths

| Origin interface | Destination | Permitted service |
| --- | --- | --- |
| Office VLAN 10 | APP01 | HTTPS TCP 443 |
| Server VLAN 20 | MON01 | TCP 1514 and 1515 |
| Server VLAN 20 | BKP01 | MinIO API TCP 9000 |
| Backup VLAN 40 | MON01 | TCP 1514 and 1515 under final firewall design |
| Management VLAN 99 | Specific infrastructure hosts | Only approved administration services |
| Office VLAN 10 | BKP01 and management services | Blocked by final policy; formal validation is reported in AG3 Prototype Testing and Validation Report |

## 5.3 Configure host VLAN interfaces

The target design is a managed VLAN capable switch and access-port and trunks are enforced at switching layer. The prototype has been implemented with an unmanaged switch (TP-Link LS108G), which means that the switch can't enforce VLAN membership, nor can it prevent an endpoint from changing its own tagging. The host interface configuration and pfSense VLAN interfaces are used to logically separate the zones, and pfSense continues to be the point of policy enforcement. The integration interface was set to the address of the Server VLAN (20) and pfSense was set as gateway for APP01. The original VMware NAT interface was only used for local build work and never used as evidence of the intended routed prototype path. This restriction is on the prototype and is not meant to imply production switch-port isolation.

Actual APP01 network configuration

APP01 was originally created and tested on VMware NAT with enp2s0 (static address 172.16.229.150/24). While team integration was in progress, the prototype network used a second interface (enp26s0) which was configured with an address of 10.20.20.10/24 and a gateway of 10.20.20.1. Checking the active interface name with ip -br address before making a change. In this draft, an additional subinterface, vlan20, is not created on APP01; instead the integration address is configured on enp26s0.

The MTU value 1400 is the integration fix which has been applied on APP01. At first, enp26s0 was using MTU 1500. The Office PC was able to access TCP 443 and HTTPS to Nextcloud was hanging. The MTU for the integration-interface interface APP01 (via the web) was reduced to 1400 and this had the effect of making normal HTTPS-export access possible again. Thus, the prototype that is demonstrated in this paper relies on MTU 1400 on enp26s0. The VMware NAT path is a local build path, and does not indicate successful integration of VLAN 20.

Stage completion check

APP01 is seen to be 10.20.20.10/24 on enp26s0 and is destined for pfSense at 10.20.20.1 and is routed on the intended routed path for prototype services. The Office PC then attempts to connect to TCP 443 and connect to the Nextcloud HTTPS page. Successful port test is followed by real browser request, since the previous MTU failure was able to make TCP (443) succeed but NOT HTTPS.

# 6 APP01 implementation guide

## 6.1 Install the application stack

APP01 is the working Nextcloud application server built by Shourab. The prototype base system consists of Ubuntu Server 24.04.4 ARM64 running on VMware Fusion Pro 26.5.2 with Apache2, MariaDB and PHP 8.3 and Nextcloud 34.0.2. This application was first filled on the local VMware NAT address 172.16.229.150 and then was plugged into the prototype network under 10.20.20.10 in VLAN 20. The build described below was actually used and the following checks need to be done when rebuilding the build (Nextcloud GmbH n.d.a; n.d.b).

Install Ubuntu Server and set the local build address to be 172.16.229.150/24 on enp2s0. Check operating system, time zone and network status prior to the web stack installation. In order to facilitate a comparison of the application and security-event times during testing, APP01 is set to use the Australia/Sydney time zone.

Set up Apache2, MariaDB and PHP 8.3 and install and secure the PHP modules necessary for Nextcloud. Create a special database user and Nextcloud database. Database credentials will not be shared or stored in GitHub or the shared report.

Install Nextcloud 34.0.2 in the path /var/www/nextcloud and perform the web configuration with the REAL data directory that is listed by the working system. The previously mentioned /srv/nextcloud-data is a working example and should not be used as the data path; please verify with occ prior to backup or monitoring/recovery configuration.

Configure Apache for Nextcloud and enable HTTPS. The prototype is using a locally generated certificate on the lab, instead of a trusted production certificate. While building and doing integrated tests locally, add both 172.16.229.150 and 10.20.20.10 to the Nextcloud trusted domains to keep the application accessible.

Validating Apache and Nextcloud prior to the integration. Ensure that your portal opens locally, you are able to log in as an administrator and a synthetic file can be uploaded and downloaded. Once the application is working locally, attach enp26s0 to the prototype network and redo the browser test via 10.20.20.10.

Stage completion check

APP01 states the Nextcloud installation and operation, Apache and MariaDB running and the actual data directory noted from occ. The application can be accessed locally at the build address and (integrated) from the Office client at 10.20.20.10 using HTTPS. The lab certificate is a prototype limitation since it is not described as a publicly trusted certificate, but rather a certificate trust prototype limitation. The browser level upload/download check is also kept since the earlier MTU issue was not found via reachability of TCP 443.

## 6.2 Configure access and application settings

Set up a test account for each individual and use the prototype names for the different groups used in the access-control evidence: All-staff, Management, Fianace, Operations, Sales, IT, Reviewers, Backup and Project-Users. The spelling "Fianace" is kept, as it is the same as the evidence implemented in the prototype.

Set up access per group so that all the users have a shared access or departmental access that they need to perform their duties. All-staff is used for general company access, Management, Fianace, Operations and Sales are separate department data, IT is technical collaboration data, Reviewers is read-only for those that have been approved, Backup is for backup related application work, and Project-Users is for prototype testing of the data. The detailed permission-mapping is still in AG3 Firewall, Segmentation and Access Control Design.

Test access model not just group list, but actual browser logins. Make sure that the required file operation is confirmed using an authorised account, and an unrelated account is used to confirm that restricted data is not available. No passwords will be recorded, along with the tested account, group, folder and result.

Enables TOTP for the main Nextcloud admin and check with a new login. Administrator TOTP has been demonstrated – MFA does not necessarily mean that MFA was enabled for all prototype users.

Enable APCu to use for Local Caching and Redis for transactional file locking. Afterwards, check if Redis is running and check Nextcloud administration warnings. The following settings were added to the base Nextcloud configuration to enhance the working APP01 configuration.

Setup Nextcloud background jobs using cron and verify that they are running as users of the web-service. This is an application cron, which is not dependent on the Restic backup schedule. Ensure that the system time zone is set to Australia/Sydney, to be able to compare the Nextcloud, backup and Wazuh timestamps during integration.

Remember to enable HTTPS, keep the two trusted-domains entries the build has set and check Nextcloud and Apache logs after configuration changes. The APP01 work failed to login and indicate that a real login was written to the source logs prior to the monitoring integration being added.

Operational checks

The APP01 stage is deemed implemented when the portal functions, the necessary prototype groups are in place and APCu/Redis and cron are configured as well as when HTTPS is enabled and administrator TOTP has been verified. Evidence from firewalls and access control devices is retained in AG3 Firewall, Segmentation and Access Control Design. Taking the ‘configuration only as proof’ approach is not reported in this case, and formal application and end-to-end validation is reported in AG3 Prototype Testing and Validation Report.

Rollback

If an APP01 change fails to enable access, revert to the previous network or application configuration that was known to work and check local HTTPS access again prior to testing via pfSense. Do not change the working configuration of 10.20.20.10 and the MTU of 1400 shown to be working, unless a controlled retest shows that it is needed. A VM checkpoint is not considered as a full Nextcloud recovery, requires a compatible application, configuration, data and database set for database or application rollback.

# 7 MON01 implementation guide

## 7.1 Install the Wazuh central components

Tanvi validates the following: MON01 address, operating system and resources. Please verify the synchronisation of time and available storage space before installing the manager, indexer and dashboard.

Use Wazuh installation assistant or installation method that is kept for the approved release. Download and check out the matching assistant before you use its all in one installation. Note when it was released and the outcome and source of the installer.

Store Administration credentials and certificate material securely. Log into the dashboard via the dashboard's approved management source and then set up named users to have the necessary dashboard access.

Check if the manager, indexer and dashboard services are running. Limit access to the deployment components that need access (indexer and API access) and don't give it to regular Office users.

The Wazuh installation process should be the same as the approved release. If an existing MON01 is rebuild or handover, this is not upgraded to an unrecorded release (Wazuh n.d.a).

## 7.2 Enrol APP01

From the Wazuh dashboard, click on agent deployment workflow. Choose APP01 operating system and architecture, manager 10.20.30.10 and set a unique name of the agent APP01.

Install/Deploy the Wazuh agent on APP01 using Package and Manager information provided in the approved MON01 release. Start the agent service and check the local log of the agent service.

Confirm the APP01 to MON01 TCP 1514 and 1515 paths. Check the dashboard to make sure that the agent listed is the one that is meant to be active and has his/her correct identity and address.

Create 1 controlled event on APP01 and verify that it shows in the dashboard, in the correct host, with the proper time and date. Event coverage is not set up by enrolment and a green connection indicator.

If one or more BKP01 agents is included in the approved monitoring design Tanvi and Faysal enrol it separately and show a new BKP01 event. In AG3 Firewall, Segmentation and Access Control Design, FT-04 and FT-05 only setting the port reachability.

## 7.3 Configure monitoring inputs

File integrity monitoring

This real Nextcloud data directory is detected using occ by Tanvi and Shourab. They first set up a dedicated synthetic folder and then the particular Company Shared path requested by the demonstration. The directory has to exist and be readable by the agent in order to create a real-time watcher.

On APP01, make a copy of /var/ossec/etc/ossec.conf. Place new monitored directory in the current syscheck configuration file and leave the other information intact.

For a separate test directory, use the example below. Monitoring a Nextcloud folder: When needed, substitute the directory that appears to be the right one with the exact approved path, do not assume that a Team Folder is in a particular numeric directory.

Restart the agent and review agent log for any configuration errors. Wait for first baseline and then create, modify and delete synthetic test file.

Identify the events on MON01, and note agent, path, event type and time. When application integration is claimed, a file that is modified via the Nextcloud browser will also be checked.

The real-time directory monitoring is set by using the syscheck settings. File-content change reporting is not enabled just to get screenshots, as it might copy sensitive contents to monitoring records. The event types shown continue to be coverage claim (Wazuh n.d.b).

Authentication and backup records

Set up the real authentication log source, and not necessarily expect Nextcloud errors to be in SSH log. Controlled failed logins are created against the service being evaluated, and are correlated with the source record and Wazuh event. A successful login will be associated with a separate test.

The backup script outputs the structured success/failure records to the backup log it chooses. Tanvi sets up the agent input and decoder/rule for that input, and then checks the one normal job and the one controlled failure. Ingestion is not guaranteed by having a network port that is permitted. A configured input and actual firewall event is required for pfSense logs to be collected.

Stage completion check

The monitoring scope shown is for fresh events from APP01. Sources/Event types with no evidence executed are not in the scope of the validated. File integrity monitoring does not necessarily equate with 100 percent ransomware detection or lossless delivery of events in the event of a monitoring failure.

# 8 BKP01 implementation guide

## 8.1 Configure MinIO storage

Faysal confirms BKP01 10.20.40.10/24 and gateway 10.20.40.1. Make sure that there is a separate repository storage directory with proper storage space and ownership for a dedicated MinIO service user.

Use the retained / verified vendor source to install the approved MinIO release. Take note of the binary/container release and the checksum. The deployment method should be the same as the build record.

Configure MinIO to store objects in the selected directory, listen on API TCP 9000 and use a different console port TCP 9001. In restricted service environment file store root credentials.

Run MinIO as per approved service unit or container configuration. Allow persistence and auto start. Before creating a bucket, look at its service log and listening ports.

From trusted administration create the bucket ag3-plains-pastoral-backups (assuming that is the approved final name). Have a separate Restic service account with a scope for this bucket and keep MinIO administration credentials off of APP01.

MinIO is provided as a storage service identity in a systemd unit or one of the approved containers and has restart behaviour and protected credentials. The actual binary or image, data directory and service name are noted. API access and console access remain different firewall paths (MinIO n.d.).

## 8.2 Verify effective permissions

Faysal is a tool that determines the read, list, write and lock operations a chosen Restic version needs. The account should be able to do normal backup/restore but not be able to administer MinIO or access other buckets. There is no write-only label to validate Restic S3 workflows.

Destructive permission checks: Make use of a disposable repository. The team notes if the APP01 account will be able to delete or overwrite repository objects. Separation of pruning and object protection or separate copy is still a recommended but not yet proven. Different VLAN and Restic encryption are NOT repository immutability.

Stage completion check

Reboot MinIO and the necessary bucket remains, APP01 is only taken to the API path, the service account is able to perform the specified actions on the repository. Unauthorised, office and server sources are still blocked. The TLS status is explicitly recorded - TCP 9000 only is not encrypted transport.

## 8.3 Connect Restic on APP01

Restic is run on APP01 and will open the connection to MinIO. Faysal and Shourab first perform a check of the disposable repository, then link the backup set to be used in the intended application. Repository passwords, storage secrets, etc., are stored in restricted files and not in source control.

Install Approved Restic release on APP01. Make /etc/ag3, and set its permissions to be restricted and a password file for which only the account running the backups has access.

Using the approved endpoint, bucket and service-account credentials, create /etc/ag3/restic.env. Choose either http or https, as per the actual MinIO transport configuration, and remember to make this choice correctly.

Execute loading of protected environment in backup execution context. Create a repository only if the bucket/prefix is brand new and empty. Use snapshots to test access of an existing repository; skip redoing init.

Log repository id, endpoint, list of buckets and snapshots without revealing the secrets. Make sure repository password can be recovered by trusted administration.

The example HTTP endpoint is for lab use that uses unverified TLS. When TLS is claimed, there must be a verified HTTPS endpoint, with a trusted certificate. The repository password is different from the MinIO key and is also needed for the recovery process (Restic Contributors n.d.a).

Required credential controls

The operator limits access to the environment and password files for the backup execution identity, and prevents passwords from being passed in arguments or shared records. Backup administration and destructive maintenance are performed with unique trusted credentials when they are used. An APP01 compromise can still reveal its local contents even if the contents of the repository are encrypted.

## 8.4 Capture a consistent Nextcloud backup

Recoverable Application Set includes the database as well as the data directory, configuration, any custom apps or themes. For the example, the paths /var/www/nextcloud, /srv/nextcloud-data and the nextcloud database are used. After creating the backup job, Faysal confirms them with Shourab from the working installation (Nextcloud GmbH n.d.c).

Set up a limited staging area that is located outside of the Nextcloud data area. Set database dump identity to have just enough access in /etc/ag3/database.cnf. Check to see if a dump can be generated without revealing its password.

Pauses any other job that can change the capture and suspend application writes by enabling maintenance mode. Note the consistency point and Time Zone. All of the paths below are obtained inside the identical managed window.

Create the database dump, check that it succeeds and is non-empty, then run the Restic backup with a stable host and tag. Record the full exit status and snapshot ID.

Disable maintenance mode upon completion of the controlled capture or upon a handled failure when it is safe to resume the application. Test the portal's functionality. This cleanup needs to be done by a script, and recorded.

Performs repository checking and a controlled restore. Keep the output from the job and the file comparison away from the protected credentials.

Backup halt on failure of database dump/backup and log it. Automated jobs need to be able to cope with failure, with concurrent execution and with "cleanup" of maintenance mode.

Normal restic check is a check of repository structure and consistency. If reading of stored data is claimed, use restic check --read-data and keep the output. Neither of the checks determines if the snapshot is prior to compromise, nor does snapshot selection rely on the incident timeline (Restic Contributors n.d.b).

## 8.5 Automate the backup and retention

Backup job requirements

The backup script has to be made with the successful sequence of captures from Section 8.4. It is required to load restricted credentials, but must not allow two runs to overlap, pause writes, create the database dump, capture the needed paths and record the result. Cleanup restores the desired application state following successful or a handled failure. Any warning or missing file as part of the backup is stored instead of being treated as a successful backup.

The Job record contains the start and finish time, timezone, consistency point, snapshot ID, captured paths, outcome and errors. It is free of any live secrets. The owner performs a normal run, an unavailable-target failure and maintenance-mode cleanup before scheduling. These are set up checks, not necessarily each recovery scenario is validated.

Use one four-hour scheduler

Determine the current scheduler. Look at root and service account crontabs, /etc/cron.d and systemd timers. Maintain the approved deployed scheduler, and turn off duplicate jobs.

If cron is the desired implementation, install one reviewed job, using absolute paths and an execution lock. The following example assumes that the script that is being reviewed is /usr/local/sbin/ag3-backup and it loads its own protected environment.

Monitor actual executions over the planned time period. Check if the snapshot has been created successfully or if it has failed to be created an appropriate message should be received; only after this is the schedule considered as implemented.

The example of cron is written as a single line. Selected wrapper/monitoring process reports a skipped locked run. If you use a Timer based deployment, it will keep that Timer and not add this cron entry.

Retention and destructive maintenance

The plan will continue to have the following approved hourly, daily, weekly and monthly rates. These are not the number of new hourlies or a precise number of snapshots. Before destructive maintenance (Restic Contributors n.d.c), the owner reviews the groups of snapshots that will be impacted and the recovery history that will be protected, and previews the retention policy.

The separately approved maintenance identity and trusted administration has been used in pruning, if available. No remaining APP01 delete capableness, retention execution, post maintenance checking are required to have recorded evidence. Backup and pruning is stopped if a compromise is suspected.

# 9 Implementation completion checks

The team integrates 1path at a time. Each completion check is performed based on the actual destination service, destination host VLAN interface and the source host. These checks are to ensure that an implementation stage is ready for formal validation and are not a substitute for the tests carried out during AG3 Prototype Testing and Validation Report. The firewall-specific acceptance evidence, the executed firewall matrix, and appendices are still present in AG3 Firewall Segmentation and Access Control Design.

A completion check only provides an answer as to whether the stage that is configured is ready for handing over for testing. The AG3 Prototype Testing and Validation Report is used to document the formal test ID, preconditions, steps taken, results observed and PASS or FAIL decision, along with a screen shot or log evidence.

| Integration check | Expected setup result | Owner |
| --- | --- | --- |
| Office to APP01 HTTPS | The Office client connects to TCP 443 and the Nextcloud portal is opened, file operations (that have been permitted) are working. The status of lab self-signed certificates is recorded. | Akib and Shourab |
| Office to BKP01 | API and console access blocked from the exact Office source | Akib |
| APP01 to MON01 | 1514/1515 reachable; APP01 enrolled; fresh event received | Tanvi and Shourab |
| BKP01 to MON01 | Approved telemetry path is part of the filter policy within a firewall – BKP01 agent monitoring not in scope unless supported in AG3 Prototype Testing and Validation Report. | Tanvi and Faysal |
| APP01 to BKP01 | API reachable; snapshot and repository objects created by the intended job | Shourab |
| Controlled restore | Selected content restored to staging and compared with expected files | Shourab |
| Restricted administration | Only the approved source and services allow management | Akib and host owner |

Successful port check will only grant reachability. It showed APP01 why an application level check is also needed: the Office PC could create TCP 443, but the HTTPS wasn't getting past until the enp26s0 MTU was reduced from 1500 to 1400. From then on, one could access the Nextcloud page and file interface via pfSense. Ping itself does not create HTTPS, Wazuh ingestion or a usable back-up, but is a routing diagnostic.

Evidence update

There are already 15 firewall cases executed for the firewall and segmentation design in AG3 Firewall Segmentation and Access Control Design. The consolidated prototype validation evidence, including the precise Office source location where appropriate, Wazuh event validation, behavior of backup and controlled recovery are located within AG3 Prototype Testing and Validation Report. This guide only documents implementation completion checks; there is no repeat of PASS evidence.

# 10 Controlled restore implementation procedure

This section only deals with technical aspects of a controlled restore. The restore is done via trusted administration to a staging directory or isolated replace VM. Incident declaration, containment, clean-point approval, credential rotation, reconnection approval and organisational recovery roles are defined in AG3 Cyber Resilience Incident Recovery Plan and are intentionally not repeated here.

From trusted administration, identify the approved repository and the snapshot selected for the restore exercise. Confirm that the snapshot contains the required paths and that the retained repository password and recovery account are available.

Make use of a different target space of adequate size. Restore to there first and keep current application state. Never cover up the one copy you have in case of investigating a fault.

Compare restored synthetic files against the original manifest, file count and SHA-256 values. Record missing or changed files. Completion of the restore process alone does not prove application recovery.

To recover the Nextcloud, Shourab creates a compatible isolated application stack and restores the corresponding data, custom applications or themes, configuration and database. Only the recovery instance can be replaced with the database.

Perform an application-level recovery exercise and verify application status, administration user login, access to folders, necessary firewall paths and new monitoring events. Formal recovery is documented in the AG3 Prototype Testing and Validation Report, and production reconnections approval is still part of the AG3 Cyber Resilience Incident Recovery Plan.

The target directory should be outside of the live Nextcloud data. The recovery of Nextcloud is based on a consistent database, data and configuration set and the imported database is set up in the isolated recovery environment beforehand. If backup is outdated, client synchronisation and data-fingerprint handling is done as per the instructions for the matching Nextcloud release (Nextcloud GmbH n.d.d).

Recovery targets and incident decisions

RPO and RTO targets, incident timeline selection, clean-point decisions and service-release authority are defined and measured in the Cyber Resilience Incident Recovery Plan and AG3 Prototype Testing and Validation Report. This implementation guide does not duplicate those governance and validation sections.

# 11 Troubleshooting guide

The operator checks what has failed first – link and VLAN tag, followed by address and route, firewall decision, listening-service or application or credential behaviour. Only changes to the observed fault are allowed and the appropriate setup check is rerun once the changes are made.

| Observed problem | Check and corrective action |
| --- | --- |
| Cannot reach own gateway | Check cable and link, VLAN ID, tagging location, IP prefix and gateway ARP. Confirm pfSense VLAN is assigned and enabled. Avoid adding a broad firewall allow rule to hide a tagging fault. |
| Gateway works but service port fails | Check route through pfSense, origin-interface rules and host firewall. On the destination, check listening ports with ss -lntp. Pair a timeout with logs; it does not identify the cause. |
| HTTPS portal fails | Check DNS name, trusted certificate, Apache configuration, PHP compatibility and Nextcloud trusted domains. Review service and application logs before reinstalling. |
| Wazuh agent is missing or events absent | Check 1514/1515, time, agent identity and local agent log. Confirm the event source or FIM path is configured and accessible. Test a fresh event after the baseline. |
| Restic cannot connect or authenticate | Confirm API 9000 rather than console 9001, endpoint scheme, bucket, service-account permission and repository password. Verify DNS and certificate trust if HTTPS is used. |
| Backup runs but recovery set is incomplete | Check captured paths and database dump success. Inspect snapshot contents and restore to staging. Correct the capture before accepting the schedule. |
| Small packets work but transfers hang | On APP01, TCP 443 succeeded but HTTPS requests stalled during Office-PC integration. Check bridge/adapter behaviour and path MTU. The demonstrated fix was changing enp26s0 from MTU 1500 to 1400, followed by a successful browser retest. Do not apply the MTU change to unrelated hosts without reproducing the symptom. |
| Repository reports an active lock | Identify the job and process that owns the lock. Do not force unlock while a backup or maintenance process remains active. |
| Management access lost after a change | Use local console or the retained recovery path to restore the previous interface or rule configuration. Verify trusted access before removing the recovery path again. |

Return to a known configuration

The owner saves logs before making another change. If a network fault occurs, it will fall back to the previous pfSense or host network configuration. All of the above features are applied in case of application faults. Objects and recovery history is saved prior to any corrective or maintenance work, due to repository failures.

# 12 Handover and production limitations

## 12.1 Handover requirements

Receiving team member is provided with final copy of address plan, versions of software, VM and adapter configuration, list of services, redacted configuration export and backup script and scheduler. Protected credentials, repository password and TOTP (time-based one-time password) recovery codes are transferred using a team agreed limited transfer method.

The operator starts pfSense, host networking and, on startup, MON01 and BKP01 and then APP01 and the clients. The handover includes the last successful job, a chosen recovery snapshot and the last restore result. The owner resets and then verifies the ongoing settings and redoes the appropriate settings check.

## 12.2 Production limitations and improvements

Limited isolation and availability with the unmanaged switch, single firewall and single service hosts. MFA is not enabled for all users via Administrator TOTP. Not all FIM and authentication events provide complete ransomware detection. Immutable storage, independent off-site copy and measured host/site recovery are not implemented unless their evidence is added to.

Production work should focus on a managed switch, a confirmed encrypted backup route, using different production and destructive credentials, securing a stand-alone backup history, and monitoring the repository administration. The following guidance is for the residual risk that they may have compromised APP01 credentials that may impact their recovery copy.

The executed evidence in AG3 Firewall and Segmentation and Access Control Design supports final firewall and access-control claims and the consolidated executed evidence in AG3 Prototype Testing and Validation Report supports overall prototype capability claims. This project repository is:

https://github.com/mahafuzfaysal2104/AG-3-Cyber-Resilience-for-an-Agribusiness-Back-Office-PG5

# References

Canonical n.d., Netplan configuration examples, viewed 1 October 2026, https://netplan.readthedocs.io/en/stable/examples/.

MinIO n.d., MinIO server project and installation guidance, viewed 16 September 2026, https://github.com/minio/minio.

Netgate n.d.a, VLAN configuration, pfSense documentation, viewed 26 September 2026, https://docs.netgate.com/pfsense/en/latest/vlan/configuration.html.

Netgate n.d.b, Rule methodology, pfSense documentation, viewed 3 October 2026, https://docs.netgate.com/pfsense/en/latest/firewall/rule-methodology.html.

Nextcloud GmbH n.d.a, Installation on Linux, Nextcloud Administration Manual, viewed 11 September 2026, https://docs.nextcloud.com/server/latest/admin_manual/installation/source_installation.html.

Nextcloud GmbH n.d.b, Example installation on Ubuntu 24.04 LTS, Nextcloud Administration Manual, viewed 6 October 2026, https://docs.nextcloud.com/server/latest/admin_manual/installation/example_ubuntu.html.

Nextcloud GmbH n.d.c, Backup, Nextcloud Administration Manual, viewed 29 August 2026, https://docs.nextcloud.com/server/latest/admin_manual/maintenance/backup.html.

Nextcloud GmbH n.d.d, Restoring backup, Nextcloud Administration Manual, viewed 9 September 2026, https://docs.nextcloud.com/server/latest/admin_manual/maintenance/restore.html.

Restic Contributors n.d.a, Preparing a new repository, restic documentation, viewed 15 September 2026, https://restic.readthedocs.io/en/stable/030_preparing_a_new_repo.html.

Restic Contributors n.d.b, Working with repositories, restic documentation, viewed 22 September 2026, https://restic.readthedocs.io/en/stable/045_working_with_repos.html.

Restic Contributors n.d.c, Removing backup snapshots, restic documentation, viewed 21 September 2026, https://restic.readthedocs.io/en/stable/060_forget.html.

Wazuh n.d.a, Quickstart, Wazuh documentation, viewed 6 October 2026, https://documentation.wazuh.com/current/quickstart.html.

Wazuh n.d.b, Basic settings for file integrity monitoring, Wazuh documentation, viewed 6 October 2026, https://documentation.wazuh.com/current/user-manual/capabilities/file-integrity/basic-settings.html.