Week 7 - Network Architecture and Physical VLAN Integration

During Week 7, I focused on moving the AG-3 network from planning into physical VLAN integration. I reviewed two alternative architectures, prepared the Cisco switch for the project networks, assigned access ports, configured the pfSense trunk and carried out the first gateway and isolation tests.

Prepared two alternative network architectures for comparison.

Used a Cisco Catalyst 3560-CX for the physical VLAN test.

Connected to the switch through PuTTY using the console interface.

Created VLAN 10 Office, VLAN 20 Server, VLAN 30 Security, VLAN 40 Backup and VLAN 99 Management for the lab-stage design.

Assigned project access ports and configured Gi0/13 as the 802.1Q trunk to pfSense.

Verified the trunk and completed the first physical integration test.

Confirmed that BKP01 could reach its own gateway at 10.20.40.1 while inter-VLAN communication still required firewall troubleshooting.

Figure 1. Week 7 - Alternative Architecture 1, simplified two-zone design.

Figure 2. Week 7 - Alternative Architecture 2, DMZ and dual-firewall design.

Figure 3. Week 7 - Cisco Catalyst 3560-CX used for the physical network test.

Figure 4. Week 7 - PuTTY serial console configured on COM9 at 9600 baud.

Figure 5. Week 7 - Cisco switch identification and physical interface status check.

Figure 6. Week 7 - VLAN table showing the project VLANs created on the Cisco switch.

Figure 7. Week 7 - Access-port configuration for the Office, Server, Security, Backup and Management lab VLANs.

Figure 8. Week 7 - Gi0/13 configured as the trunk connection to pfSense.

Figure 9. Week 7 - Trunk verification showing the allowed and active AG-3 VLANs.

Figure 10. Week 7 - Physical cabling during the team integration test.

Figure 11. Week 7 - BKP01 gateway test. The host reached 10.20.40.1 while other inter-VLAN tests still required firewall checking.

Week 8 - pfSense Connectivity, Interface Checks and Design Refinement

During Week 8, I continued the pfSense integration work and checked the project VLAN interfaces in the home-lab environment. I also tested Internet access and server connectivity. At the same time, I developed the selected head-office network design in more detail so the logical design could match the project requirements.

Confirmed the pfSense VLAN interfaces and gateway addressing from the console.

Confirmed that pfSense could reach the Internet by pinging 8.8.8.8 with no packet loss.

Checked the Windows test computer addressing and local pfSense LAN connection.

Tested the Server VLAN gateway from the APP01 side and obtained a successful 10.20.20.1 ping after earlier failed attempts.

Refined the final head-office design and removed unnecessary complexity from the final documentation.

Expanded the design to include Office, Server, Security, Backup and Guest Wi-Fi networks with detailed device addressing.

New firewall rule added that allows each subnet to ping the firewall itself for testing purposes.

Figure 12. Week 8 - pfSense console showing the project VLAN interfaces and gateway addresses.

Figure 13. Week 8 - pfSense Internet connectivity test to 8.8.8.8 showing 0% packet loss.

Figure 14. Week 8 - Windows ipconfig output used to verify the local test-machine addressing during integration.

Figure 15. Week 8 - APP01-side test showing successful communication with the Server VLAN gateway 10.20.20.1.

Detailed network-design progress

The selected design was developed as a simplified single-head-office network. The final design documentation was kept separate from the smaller implementation environment. The final simplified design uses Office, Server, Security, Backup and Guest Wi-Fi networks. The Management network that appears in some earlier lab screenshots was later removed from the final simplified design.

Figure 16. Week 8 - Final simplified AG-3 head-office overview without the Management subnet.

Figure 17. Week 8 - Detailed Office gateway design with PC 1 to PC 30, IP phones and printers assigned individual IP addresses.

Figure 18. Week 8 - Detailed Server gateway design showing APP01 and AUTH01.

Figure 19. Week 8 - Detailed Backup gateway design showing BKP01 and NAS01.

Figure 20. Week 8 - Detailed Guest Wi-Fi gateway design with the DHCP client range.

I focused on the Security VLAN because the connected security endpoint could not initially reach its own gateway at 10.20.30.1. I used routing, interface and packet-capture checks to identify whether the problem was on pfSense, the endpoint or the VLAN path.

Checked the pfSense routing table and confirmed directly connected routes for the project VLANs.

Captured the initial failure where the Security endpoint at 10.20.30.10 could not reach 10.20.30.1.

Checked the pfSense Security interface re0.30 and confirmed VLAN 30, 10.20.30.1/24 and active status.

Used tcpdump on the pfSense parent interface to confirm tagged VLAN 30 ARP traffic.

Confirmed ARP requests and replies between 10.20.30.10 and 10.20.30.1.

Retested from the Security endpoint and successfully reached 10.20.30.1 with 0% packet loss.

Figure 21. Week 8 - pfSense routing table showing the connected project VLAN routes.

Figure 22. Week 8 - Initial Security VLAN failure. The endpoint at 10.20.30.10 could not reach gateway 10.20.30.1.

Figure 23. Week 8 - pfSense ifconfig output confirming re0.30, VLAN 30, address 10.20.30.1/24 and active link status.

Figure 24. Week 8 - pfSense tcpdump showing tagged VLAN 30 ARP requests and replies between 10.20.30.10 and 10.20.30.1.

Figure 25. Week 8 - Final Security VLAN gateway test showing successful replies from 10.20.30.1 with 0% packet loss.

Figure 26. Week 8 – New firewall rule added that allows each subnet to ping the firewall itself for testing purposes.

Current Detailed Address Plan

The detailed head-office design now uses individual addresses instead of general labels such as PC, printer or camera. The main planned address allocations are shown below.

| Network | Device / range | Address | Purpose |
| --- | --- | --- | --- |
| Office | Gateway | 10.20.10.1/24 | Office default gateway |
| Office | PC 1 - PC 30 | 10.20.10.11 - 10.20.10.40 | Employee computers |
| Office | IP Phone 1 - 10 | 10.20.10.41 - 10.20.10.50 | Office voice devices |
| Office | Printer 1 - 3 | 10.20.10.51 - 10.20.10.53 | Shared printers |
| Server | Gateway | 10.20.20.1/24 | Server default gateway |
| Server | APP01 | 10.20.20.10/24 | Business application / Nextcloud |
| Server | AUTH01 | 10.20.20.12/24 | Authentication service |
| Security | Gateway | 10.20.30.1/24 | Security default gateway |
| Security | MON01 | 10.20.30.10/24 | Security monitoring |
| Security | NVR01 | 10.20.30.20/24 | CCTV recording |
| Security | ACC01 | 10.20.30.30/24 | Access-control controller |
| Security | CCTV 1 - 10 | 10.20.30.101 - 10.20.30.110 | IP cameras |
| Security | RFID 1 - 4 | 10.20.30.121 - 10.20.30.124 | RFID access readers |
| Backup | Gateway | 10.20.40.1/24 | Backup default gateway |
| Backup | BKP01 | 10.20.40.10/24 | Backup server |
| Backup | NAS01 | 10.20.40.11/24 | Network attached storage |
| Guest Wi-Fi | Gateway | 10.20.50.1/24 | Guest default gateway |
| Guest Wi-Fi | DHCP clients | 10.20.50.100 - 10.20.50.199 | Guest devices |